Configuration of switched-mode power supply without galvanic isolation

The behavior of the convertors is described by the transfer function M depending on the switching ratio D.

These basic configurations are suitable for output power	100 W — 150W

Operating frequency range					30 kHz do 1 MHz.

Control circuits							PWM modulation

Constant t on

Constant t off

Using integrated circuits, they have a low number of components and a "simple circuit"

Voltage transfer as function of duty cycle for various topologies

D = t on / T , M = Uout / Uin

# Step down (buck)

Explanations:

Zap = ON

Vyp = OFF

Roste = rise

Klesá = fall

STEP DOWN

Wa=12LIta2    ;     Wb=12LItb2

For Continuous Mode:    Wa=Wb

ILta=1Lota(UIN-UOUT)∙dt= 1L(UIN-UOUT)∙ta

ILtb=1LotbUOUT∙dt= 1LUOUT∙tb

1LUIN∙ta-1LUOUT∙ta=1LUOUT∙tb

UIN∙ta=UOUTtb+ta  / ta+tb=T

UIN∙ta=UOUT∙T

UOUT=UIN∙taT	/ taT = D		DUTY CYCLE

UOUT=UIN∙D

Transfer function	 M=UOUTUIN

M = D

In continuous mode, the voltage conversion is given only by the input voltage UIN and the duty cycle.

In discontinuous mode, the conversion is given: UIN; D; output load, magnitude of induktance

For discontinuous mode:

UOUT=UIN∙21+4∙KD2

K=2∙LRz∙T

INDUCTANCE VALUE

Discontinuous current - Critical current Iocrit

Iocrit is define as the minimum current to maintain continuous conduction mode.

Iocrit=∆IL2	IL=otIoUOUT∙ dt                  /  IL=∆IL

/   ∆IL=2IOCRIT

2IOCRIT=1LUOUT∙tb  / tb=tOFF

Iocrit=UOUT∙tOFF2L

Lmin=UOUT∙tOFF2IOCRIT

CAPACITOR VALUE:

→ Ic= ∆IL4

Uc=1coT2ictdt=1c∙ ∆IL4∙T2=∆IL∙T8C=∆IL8Cf

∆Uc=∆IL8Cf   →     Cmin=∆IL8f∙∆Uc		for continuous Mode

→     Cmin=IOUT∙(1-IOUTmax∆IL)28∙∆UOUT   for discontinuous Mode

→	ESRnax=∆Uc max∆IL

UESRt=ict∙ESR

UOUT=Uc+UESR

# Step up - Boost

Explanations:

Zap = ON

Vyp = OFF

Roste = rise

Klesá = fall

Voltage curves Ux and current IL1 for three operation modes.

BOOST = STEP-UP

Wta=12LI(ta)2		Wtb=12LI(tb)2

For continuous mode:

IL(ta)=1LotaUINdt=1LUIN∙ta

IL(tb)=1LotbULdt

UL=UOUT-UIN	/UOUT=UIN+UL  /

IL(tb)=1Lotb(UOUT-UIN)∙dt=1LUOUTtb-1LUINtb

↓

1LUINta=1LUOUTtb-1LUINtb

UINta+UINtb=UOUTtb

UIN(ta+tb)tb=UOUT	/    T=ta+tb

UOUT=UIN∙Ttb

D=taT     ;      D=tata+tb      ; 				D∙ta+D∙tb=ta

M=UOUTUIN						Dta+Dtb=ta

tb=ta(1-D)D

UIN(ta+tb)tb=UOUT

UOUTUIN=(ta+tb)tb

M=ta+ ta(1-D)Dta(1-D)D=taD+ta-taDDta-taDD

M=taD∙Dta(1-D)

M=11-D

For discontinuous mode:

UOUT=UIN∙1+1+4∙D2K2

K=2LRT

INDUCTANCE VALUE

Discontinuous current - Critical current Iocrit (calculated independently on I0)

Io crit=∆IL2

IL=1LotaUINdt=   /IL=∆IL      ∆IL=2Iocrit

Iocrit=12Lmin∙UIN∙ta

Lmin=12Io critUIN∙tON

Lmin=UIN∙tON2Iocrit

# Buck – boost

Explanations:

Zap = ON

Vyp = OFF

Roste = rise

Klesá = fall

BUCK-BOOST

Wta=12LII(ta)2

IL(ta)=1LotaUINdt=UIN∙taL

IL(tb)=-1LotbUOUTdt=-UIN∙tbL

UIN∙taL=UOUT∙tbL

UOUT=-UIN∙tatb

M=UOUTUIN

D=taT=tata+tb

Dta+Dtb=ta

tb=ta(1-D)D

M=-tata (1-D)D

M=-D(1-D)	→ for continuous mode

For discontinuous mode:

UOUT=UIN∙DK		K=Z∙LR∙T

# Inverting converter Ćuk

Explanations:

Zap = ON

Vyp = OFF

Roste = rise

Klesá = fall

Continuation of waveforms

ČUK	SLOBODAN ČUK / CALIFORNIA INSTITUTE OF TECHNOLOGY 1976

L1=L2

Wta=Wtb

ILta=1LotaUIN∙dt= 1LUIN∙ta

ILtb=-1LotbUOUT∙dt= -1LUOUT∙tb

UIN∙ta=-UOUT∙tb

UOUT=-UIN∙tatb

D=taT

D=tata+tb

tb=ta(1-D)D

M=UOUTUIN

M=-tata(1-D)D

M=-ta1∙Dta1-D

M=-D(1-D)

# SEPIC Converter

Continuation of waveforms

MĚNIČ SEPIC

Wta=Wtb

ILta=1LotaUIN∙dt= 1LUIN∙ta

ILtb=1LotbUOUT∙dt= 1LUOUT∙tb

UINta=UOUT∙tb

UOUTUIN=tatb

UOUT=UIN∙tatb

D=tata+tb

Dta+Dtb=ta

tb=ta1-DD

M=tatb

M=ta1∙Dta(1-D)

M=D1-D