Example 1:

Calculate initial short-circuit current at the 400 V switchboard:

- For supplying only from upstream network, without or with motors.

- For supplying from upstream network and local generator, without or with motors.

Parameters:

Upstream network: Un = 22 kV; Sk′′  = 250 MVA; c = 1,1

Transformer T1: SnT1 =  40 MVA; uk = 7 %

Transformer T2: SnT2 =  1 MVA; uk = 6 %

Generator: SnG =  10 MVA; xd “= 12 %

Cable: X = 0,15 Ω/km; l = 1000 m

Motors M1: Pn1 = 300 kW; cosϕM = 0,8; ηM = 0,95; iz = 5

Example 2:

Calculate initial short-circuit current at the 400 V switchboard:

Parameters:

Upstream network: Un = 22 kV; Sk′′  = 250MVA; c = 1,1; XS = 0,995 ZS ; RS = 0,1 XS

Transformer T1: SnT1 =  10 MVA; uk = 7 %; tr = 22/6,3; ΔPk = 25 kW

Transformer T2: SnT2 =  1 MVA; uk = 6 %; tr = 6/0,4; ΔPk = 10,5 kW

Motors M2: 8 ks; PnM2 = 200 kW; UnM = 6 kV; iz = 5; cosϕM = 0,8; ηM = 0,95; kS  β = 0,75 (i.e. only 6 from 8 motors are running)

Motors M1:

2 x Pn1 = 100 kW; 3 x Pn2 = 50 kW (1 back-up motor, i.e. only 2 from 3 motors are running); UnM = 400 V; iz = 6; cosϕM = 0,8; ηM = 0,95

Example 3:

Calculate initial short-circuit current at end of MV cable, without or with reactance coil:

Upstream network: Un = 110 kV; Sk′′  = 2500 MVA; c = 1,1

Transformer: T1: SnT1 =  40 MVA; uk = 13 %

Generator: SnG =  25 MVA; xd “= 14 %

Reactance coil: UnR = 6 kV; In =  1000 A; ur = 8 %

Cable: X = 0,1575 Ω/km; l = 1000 m

Example 4:

Calculate initial short-circuit current at end of MV cable, without or with reactance coil:

Network S:

Un = 110 kV; Sk′′ = 3500 MVA; c = 1,1

Transformers T1, T2:

SnT =  31,5 MVA; uk = 10,5 %; 		110/23 kV; ΔPk = 175 kW

Reactance coilr Re:

ukR = 12 %; Spr = 10 MVA

Cable K:

RL = 0,32 Ω/km; XL = 0,12 Ω/km-1; 	l = 6 km